The Town of Salina Landfill site, approximately 55 acres in size, is located in the Town of Salina, Onondaga County, New York. It is designated a Class 2 Inactive Hazardous Disposal Waste Site by the New York State Department of Environmental Conservation (NYSDEC) (New York Registry No. 7-34-036). The Site is bounded by the New York State Thruway to the north and by Route 11 (Wolf Street) to the east. An Onondaga County Resource Recovery Agency Transfer Station is located immediately to the west of the landfill. Ley Creek, a Class B stream, runs through the approximate eastern half of the Site and along the southern border of the approximate western half of the Site. The eastern half of the Site is bounded to the south by the banks of a separate tributary, known as the Old Ley Creek Channel (OLCC). A portion of Ley Creek was moved in the early 1970s to its current location. Landfilled materials have been identified both north of Ley Creek and south of Ley Creek in the land area located between the current Ley Creek and the OLCC, (i.e., north and south of Ley Creek). The sediments, surface waters and banks of Ley Creek under and downstream of the Route 11 bridge, as well as the sediments, surface waters, and banks of the OLCC are collectively a separate Class 2 New York State inactive hazardous waste disposal site known as the "Old Ley Creek Channel Site" (Site Number 734074).

In 1996, NYSDEC designated the Town of Salina Landfill as a Class 2 Inactive Hazardous Waste Site. This designation means that NYSDEC considers the Site a significant threat to human health and/or the environment, which requires remedial action. This Site was designated a subsite to the Onondaga Lake Superfund Site in June 1997 by NYSDEC and EPA, due to the fact that Site contaminants had migrated to Ley Creek, which flows into the lake. In 1997, representatives from NYSDEC collected three sediment samples from the OLCC. The results of that sampling show that detectable concentrations of VOCs, SVOCs, and PCBs are present in Old Ley Creek Channel. The portion of Ley Creek adjacent to the landfill is not part of the Site due to the presence of upstream sources of contamination that need to be addressed. Upstream contaminated surface water and sediments in Ley Creek are currently being investigated under an RI/FS for the General Motors Former Inland Fisher Guide Facility and Ley Creek Deferred Media subsite of the Onondaga Lake site. The sediments, surface waters and banks of Ley Creek under and downstream of the Route 11 Bridge as well as the sediments, surface waters and banks of the OLCC are collectively being addressed as the "Old Ley Creek Channel Site," which is a separate Class 2 New York State inactive hazardous waste disposal site (Site Number 734074).

From: EPA | Envirofacts Warehouse | CERCLIS Page 6 of 9 http://oaspub.epa.gov/enviro/cerclis_web.description_report?pgm_sys_id=NYD986913580 11/5/2009

